Gel Purification of RNA

- Pour a 6% acrylamide/7M urea gel (add 550ul 10% APS and 55ul Temed to 55ml of mix)

- Prerun gel at 40W, 30’

- Heat RNA to 90º, 5’ in an equal volume of denaturing buffer (90% formamide, 1X TBE, BPB, XC)

- Remove urea from wells, load samples

- Run at 40W

- For radioactive RNA, wrap in saran wrap and expose to film 3-30’ (mark film with high energy pen

- For nonradioactive samples use UV light to detect RNA

- Excise the RNA with a clean razor blade. Use RNase Zap treated forceps to place in eppie.

- Add 500-600ul of Elution buffer	stock	final (ul)

10mM Tris-HCl pH 8			1M	6

1mM EDTA				0.5M	1.2

0.75M NH4OAc			10M 	45

0.1% SDS				20%	3

DEPC H2O					544.8

- Rock overnight at 4º

- Transfer liquid to a fresh eppie, add NH4OAc to 2M (~80ul) and 1 volume of cold isopropanol.  Precipitate at -20 º at least an hour.

- Spin 30-45’ and remove super. Wash pellet with 500ul 70% Ethanol. Spin 5’. Dry briefly and resuspend pellet in 25ul of DEPC water or TE. Store hot RNA at -20 º and cold RNA at -80 º.