Protein Purification

- Weigh cell pellet then resuspend in 2mL/g Buffer A- for GST or C- for Talon with lysozyme (1:1000 dil) and DNaseI (1:1000 dil) with 1mM PMSF + DTT- for GST or 5mM β2Me- for Talon.

- Sonicate 1X on ice, 10x pulse at 50% duty cycle, 5-6 output control.

- Add NaCl to 1M and sonicate 3x.

- Spin 25,000xg 30 min 4°C, save 10ul of lysate and 1ul pellet+9ul buffer for gel.

- Add 1:20 dil of 10% PEI (0.5% final) drop wise and mix 20 min at 4°C.

- Spin 25,000xg 30 min 4°C.

- Optional Precipitation: Add sat NH3SO4 to 65% drop wise while gently mixing, then spin 25,000xg 30 min 4°C.  Weigh pellet and resuspend in 2mL/g Buffer A- for GST or C- for Talon without PMSF (for GST tag removal if desired).

- Add 1-2mL of slurry beads (Glut-Ag for GST or Talon for His) and rock for 2 hrs at 4°C.  Glut-Ag 1mL beads (2ml slurry)=6ug protein, Talon1mL beads (2ml slurry)=3mg.

- Spin 1500 rpm 4°C 4 mins, save sup and wash beads, saving each wash:

GST Glut-Ag - 3x Buffer A, 2x Buffer B, 2x Buffer B + 1M NaCl, add beads to column and wash through with Buffer B.

Talon His - 4x Buffer C, 2X Buffer C + 1M NaCl, add beads to column and wash through with Buffer C.  Can add 10mM imidazole to wash to get rid of non-specific binding.

- Elute with 500ul - 1mL fractions:

GST Glut-Ag - Buffer B + 10mM glutathione, make 100mM glutathione fresh from power.

Talon His – Buffer C + 150mM imidazole, make fresh from power.

- Check protein fractions with mini Bradford assay (2ul dye to 8ul elution), pool positive protein fractions and dialyze O/N at 4°C.

Buffers

Buffer A			Buffer B			Buffer C

50mM Tris pH=8.0		50mM Tris pH=8.7		50mM Tris pH=8.0			250mM NaCl			200mM NaCl			250mM NaCl				5mM EDTA			1mM EDTA																				Add these fresh:											10mM DTT			10mM DTT			5mM β2Me				1mM PMSF							1mM PMSF